A REPORT ON

DRIVER FOR BME280

BY

JEEVARAAM K           2020H1400216H

RISHI S PHAYE           2020H1400230H

M.E. EMBEDDED SYSTEMS

Prepared in partial fulfilment of the

(G547)

DEVICE DRIVERS

BIRLA INSTITUTE OF TECHNOLOGY & SCIENCE, PILANI

(DECEMBER 2021)

LIST OF FIGURES:

Figure 1: BME280 sensor…………………………………………………………..4

Figure 2: Memory map for BME280……………………………………………………..5

Figure 3: Compensation parameter storage,naming and data type……………………….6

Figure 4: Connection with BME280 with raspberry pi 4…………………………………7

Figure 5: Pin diagram of raspberry pi	…………………………………………………….7

Figure 6: Actual connection with BME280……………………………………………….8

CONTENTS

1. Summary……………………………………………….………………....4

2. Hardware design……………………………………………………...…..7

3. Kernel Space Driver and its build process………………………………..9

4. User space application build process and usage…………………………17

5. IOCTL Header file……………………………………………………….20

6. Makefile……………………………………………………………….…21

7. Makefile commands……………………………………………………...23

SUMMARY:

- This project collects the data(temperature, humidity and pressure) from the sensor BME280 and stores it in a log file in given intervals.

- The user space application is capable of receiving data from the kernel using IOCTL call

- The received data is logged into a log file in the same directory.

- In kernel, once the IOCTL call is received the kernel initiates the I2C read from the sensor applies compensation algorithm to the raw values and transfers them to the user space request; the process is same for all the three parameters.

Figure 1: BME280 sensor

Register description:

Figure 2 :Memory map for BME280

- The entire communication with the device is performed by reading from and writing to registers, the above memory map is used for all those purposes.

- The Following points must be noted:

- For humidity the register address 0xFE and 0xFD are being used to get the raw data value.

- For temperature the register address 0xFC, 0xFB and 0xFA are being used to get the raw data.

- For pressure the register address starting 0xF9, 0xF8 and 0xF7 are being used to get the raw data.

- Config(0xF5) and ctrl_meas(0xF4) are configured with respective parameters for proper sensing and operation of sensor.

READING PARAMETERS:

Figure 3: Compensation parameter storage,naming and data type

- The parameter are programmed inside the non-volatile memory during production and we can’t alter it.

- Each compensation word is a 16-bit signed or unsigned integer value stored in 2’s complement form.

- 2 words must always be combined in order to represent the compensation word, as the memory is organized into 8-bit words.

- These compensation parameters are read by the kernel during initialization and kept in global space of kernel module for further calculations during runtime.

HARDWARE  SCHEMATIC:

Figure 4: Connection with BME280 with raspberry pi 4

Figure 5: Pin diagram of raspberry pi

The connections for our code will be as explained below:

- 3.3V is given to pin 1 of Raspberry pi and the Vdd pin of BME280

- Pin number 3 of Raspberry Pi is connected to the SDA pin of BME280.

- Pin number 5 of Raspberry Pi is connected to the SCL pin of BME280.

- Pin number 6 of Raspberry Pi and GND pin of  BME280 are connected to ground.

Figure 6: Actual connection with BME280.

Kernel Space Driver and it’s build process:

/* General Includes for Kernel Compilation */

#include <linux/module.h>

#include <linux/init.h>

#include <linux/fs.h>

#include <linux/version.h>

#include <linux/cdev.h>

#include <linux/uaccess.h>

#include <linux/slab.h>

#include <linux/i2c.h>

#include <linux/kernel.h>

/* IOCTL Call Definition Header */

#include "config.h"

#define DRIVER_NAME "bme280" //Name of the driver to be created

#define DRIVER_CLASS "bme280Class" //Name of the class to be used for driver

/* User-defined data types for Pressure compensation algorithm */

typedef uint32_t BME280_U32_t;

typedef int32_t BME280_S32_t;

static struct i2c_adapter * bme_i2c_adapter = NULL; //I2C Adapter

static struct i2c_client * bme280_i2c_client = NULL; //I2C Client

/* Variables for holding Trim parameters from the sensor */

int32_t calib_T1,calib_T2,calib_T3,calib_Tfine;

int32_t calib_P1,calib_P2,calib_P3,calib_P4,calib_P5,calib_P6,calib_P7,calib_P8,calib_P9;;

int32_t calib_H1,calib_H2,calib_H3,calib_H4,calib_H5,calib_H6;

/* Driver Metadata */

MODULE_AUTHOR("Jeevaraam K, Rishi S Phaye");

MODULE_LICENSE("GPL");

MODULE_DESCRIPTION("BME280 Sensor Kernel Driver");

#define I2C_BUS_USED       1 //I2C Bus used for Communication

#define BME280_SENSOR_NAME       "BME280" //Sensor name for Metadata

#define BME280_SLAVE_ADDRESS    0x77 //I2C Address of Sensor used

/* Device ID Structure Initialization */

static const struct i2c_device_id bme280_dev_id[]={

{ BME280_SENSOR_NAME, 0},

{ }

};

/* I2C Driver Structure Initialization */

static struct i2c_driver bme280_driver = {

.driver = {

.name = BME280_SENSOR_NAME,

.owner = THIS_MODULE

}

};

/* Board and Device Mapping Structure */

static struct i2c_board_info bme280_i2c_board_info = {

I2C_BOARD_INFO(BME280_SENSOR_NAME, BME280_SLAVE_ADDRESS)

};

static dev_t bme280_device_number; //Major and Minor Number holding variable

static struct class *bme280_class; //Sensor class variable

static struct cdev bme280_device; //Char device for Sensor

/*BME280 Compensation Routines */

/* Compensation Routine for reading and Converting Raw Sensor value to Humidity Values */

static uint32_t read_humidity(void)

{

int32_t var1;

int32_t var2;

int32_t var3;

int32_t var4;

int32_t var5;

uint32_t humidity;

uint32_t humidity_max = 102400;

uint8_t lsb,msb;

uint32_t current_humidity;

/*Read Raw values from sensor */

lsb = i2c_smbus_read_byte_data(bme280_i2c_client, 0xFE);

msb = i2c_smbus_read_byte_data(bme280_i2c_client, 0xFD);

current_humidity = ((uint32_t)msb<<8) | ((uint32_t)lsb);

/* Compensation Algorithm */

var1 = calib_Tfine - ((int32_t)76800);

var2 = (int32_t)(current_humidity * 16384);

var3 = (int32_t)(((int32_t)calib_H4) * 1048576);

var4 = ((int32_t)calib_H5) * var1;

var5 = (((var2 - var3) - var4) + (int32_t)16384) / 32768;

var2 = (var1 * ((int32_t)calib_H6)) / 1024;

var3 = (var1 * ((int32_t)calib_H3)) / 2048;

var4 = ((var2 * (var3 + (int32_t)32768)) / 1024) + (int32_t)2097152;

var2 = ((var4 * ((int32_t)calib_H2)) + 8192) / 16384;

var3 = var5 * var2;

var4 = ((var3 / 32768) * (var3 / 32768)) / 128;

var5 = var3 - ((var4 * ((int32_t)calib_H1)) / 16);

var5 = (var5 < 0 ? 0 : var5);

var5 = (var5 > 419430400 ? 419430400 : var5);

humidity = (uint32_t)(var5 / 4096);

if (humidity > humidity_max)

{

humidity = humidity_max;

}

return humidity;

}

/* Compensation Routine for reading and Converting Raw Sensor value to Pressure Values */

static uint32_t read_pressure(void)

{

/*int32_t var1;

int32_t var2;

int32_t var3;

int32_t var4;

uint32_t var5;*/

uint32_t current_pressure;

uint8_t lsb,msb,xlsb;

BME280_S32_t var1, var2;

BME280_U32_t p;

/* Read raw values from Sensor */

lsb = i2c_smbus_read_byte_data(bme280_i2c_client,0xF8);

msb = i2c_smbus_read_byte_data(bme280_i2c_client,0xF7);

xlsb = i2c_smbus_read_byte_data(bme280_i2c_client, 0xF9);

current_pressure = (((uint32_t)msb)<<12) | ((uint32_t)lsb<<4) | ((uint32_t)xlsb>>4);

// current_pressure = ( (current_pressure<<4) | ( (uint32_t) (xlsb&0xF0) >>4 ) );

printk("Pressure:%x\n",current_pressure);

/* Compensation Algorithm */

var1 = (((BME280_S32_t)calib_Tfine)>>1) - (BME280_S32_t)64000;

var2 = (((var1>>2) * (var1>>2)) >> 11 ) * ((BME280_S32_t)calib_P6);

var2 = var2 + ((var1*((BME280_S32_t)calib_P5))<<1);

var2 = (var2>>2)+(((BME280_S32_t)calib_P4)<<16);

var1 = (((calib_P3 * (((var1>>2) * (var1>>2)) >> 13 )) >> 3) + ((((BME280_S32_t)calib_P2) * var1)>>1))>>18;

var1 =((((32768+var1))*((BME280_S32_t)calib_P1))>>15);

if (var1 == 0)

{

return 0; // avoid exception caused by division by zero

}

p = (((BME280_U32_t)(((BME280_S32_t)1048576)-current_pressure)-(var2>>12)))*3125;

if (p < 0x80000000)

{

p = (p << 1) / ((BME280_U32_t)var1);

}

else

{

p = (p / (BME280_U32_t)var1) * 2;

}

var1 = (((BME280_S32_t)calib_P9) * ((BME280_S32_t)(((p>>3) * (p>>3))>>13)))>>12;

var2 = (((BME280_S32_t)(p>>2)) * ((BME280_S32_t)calib_P8))>>13;

p = (BME280_U32_t)((BME280_S32_t)p + ((var1 + var2 + calib_P7) >> 4));

return p;

static int32_t read_temperature(void)

{

int32_t var1,var2;

uint8_t aTemp,bTemp,cTemp;

int32_t temperature;

int32_t finalTemp;

/* Read raw values from Sensor */

aTemp = i2c_smbus_read_byte_data(bme280_i2c_client, 0xFA);

bTemp = i2c_smbus_read_byte_data(bme280_i2c_client, 0xFB);

cTemp = i2c_smbus_read_byte_data(bme280_i2c_client, 0xFC);

/* Compensation Algorithm */

temperature = (((int32_t)aTemp) << 12) | (((int32_t)bTemp) << 4) | (((int32_t)cTemp) >> 4);

var1 = (int32_t)((temperature / 8) - ((int32_t)calib_T1 * 2));

var1 = (var1 * ((int32_t)calib_T2)) / 2048;

var2 = (int32_t)((temperature / 16) - ((int32_t)calib_T1));

var2 = (((var2 * var2) / 4096) * ((int32_t)calib_T3)) / 16384;

calib_Tfine = var1 + var2;

finalTemp = ((var1 + var2) * 5 + 128) / 256;

printk("Temp:%d\n",finalTemp);

printk("Tfine:%d\n",calib_Tfine);

return finalTemp;

}

/* Direct Call Read routine for I2C device */

static ssize_t bme280_driver_read(struct file *File, char __user *user_buffer, size_t count, loff_t *offs)

{

int to_copy, not_copied, delta;

char out_string[30];

int32_t temperature;

int32_t pressure;

int32_t humidity;

to_copy = min(sizeof(out_string), count);

/* Read all 3 values */

temperature = read_temperature();

pressure = read_pressure();

humidity = read_humidity();

/* Transfer the read values */

snprintf(out_string, sizeof(out_string), "T:%d,P:%d,H:%d\n",temperature,pressure,humidity);

not_copied = copy_to_user(user_buffer, out_string, to_copy);

delta = to_copy - not_copied;

return delta;

}

/* IOCTL Call routine */

long ioctl_dev(struct file *file, unsigned int ioctl_num, unsigned long ioctl_param)

{

switch(ioctl_num)

{

/* IOCTL Call for Temperature */

case IOCTL_TEMPERATURE:

put_user(read_temperature(),(int32_t*)ioctl_param);

break;

/* IOCTL Call for Pressure */

case IOCTL_PRESSURE:

put_user(read_pressure(),(int32_t*)ioctl_param);

break;

/* IOCTL Call for Humidity */

case IOCTL_HUMIDITY:

put_user(read_humidity(),(int32_t*)ioctl_param);

break;

}

return 0;

}

/* Driver open routine */

static int bme280_driver_open(struct inode *deviceFile, struct file *instance)

{

printk("Driver Open\n");

return 0;

}

/* Driver close routine */

static int bme280_driver_close(struct inode *deviceFile, struct file *instance)

{

printk("Driver Close\n");

return 0;

}

/* File Operations structure for Open, Close, Read and IOCTL routines linkage */

static struct file_operations fops  = {

.owner = THIS_MODULE,

.open = bme280_driver_open,

.release = bme280_driver_close,

.unlocked_ioctl = ioctl_dev,

.read = bme280_driver_read,

};

/* Driver Initialization Routine */

static int __init bme280Init(void)

{

int ret =-1;

int32_t tempH4a,tempH4b,tempH5a;

u8 id;

printk("Driver Init\n");

/* Allocating Random Major number from Linux with one device */

if ( alloc_chrdev_region(&bme280_device_number, 0, 1, DRIVER_NAME) < 0)

{

printk("Device Number could not be allocated!\n");

}

printk("Device Number %d was registered\n", bme280_device_number);

/* Create Device Class */

if ((bme280_class = class_create(THIS_MODULE, DRIVER_CLASS)) == NULL)

{

printk("Device Class can not be created!\n");

goto ClassError;

}

/* Create Device file */

if (device_create(bme280_class, NULL, bme280_device_number, NULL, DRIVER_NAME) == NULL)

{

printk("Can not create device file!\n");

goto FileError;

}

/* Initialize Device file */

cdev_init(&bme280_device, &fops);

/* register device to kernel */

if (cdev_add(&bme280_device, bme280_device_number, 1) == -1)

{

printk("Registering of device to kernel failed!\n");

goto KernelError;

}

/* Locking I2C Bus Adapter */

bme_i2c_adapter = i2c_get_adapter(I2C_BUS_USED);

if(bme_i2c_adapter != NULL)

{

/* I2C Client Static Link */

bme280_i2c_client = i2c_new_client_device(bme_i2c_adapter, &bme280_i2c_board_info);

if(bme280_i2c_client != NULL)

{

/* Add I2C Driver */

if(i2c_add_driver(&bme280_driver) != -1)

{

ret = 0;

}

else

printk("Can't add driver...\n");

}

/* Releasing I2C Bus Adapter */

i2c_put_adapter(bme_i2c_adapter);

}

printk("BME280 Driver Init\n");

/* Read Device ID for Communication Verification */

id = i2c_smbus_read_byte_data(bme280_i2c_client, 0xD0);

printk("ID: 0x%x\n",id);

/* Read Trim Values from the Sensor and store in Global Kernel Variables */

calib_T1 = i2c_smbus_read_word_data(bme280_i2c_client,0x88);

calib_T2 = i2c_smbus_read_word_data(bme280_i2c_client,0x8A);

calib_T3 = i2c_smbus_read_word_data(bme280_i2c_client,0x8C);

calib_P1 = i2c_smbus_read_word_data(bme280_i2c_client,0x8E);

calib_P2 = i2c_smbus_read_word_data(bme280_i2c_client,0x90);

calib_P3 = i2c_smbus_read_word_data(bme280_i2c_client,0x92);

calib_P4 = i2c_smbus_read_word_data(bme280_i2c_client,0x94);

calib_P5 = i2c_smbus_read_word_data(bme280_i2c_client,0x96);

calib_P6 = i2c_smbus_read_word_data(bme280_i2c_client,0x98);

calib_P7 = i2c_smbus_read_word_data(bme280_i2c_client,0x9A);

calib_P8 = i2c_smbus_read_word_data(bme280_i2c_client,0x9C);

calib_P9 = i2c_smbus_read_word_data(bme280_i2c_client,0x9E);

calib_H1 = i2c_smbus_read_byte_data(bme280_i2c_client,0xA1);

calib_H2 = i2c_smbus_read_word_data(bme280_i2c_client,0xE1);

calib_H3 = i2c_smbus_read_byte_data(bme280_i2c_client,0xE3);

tempH4a = i2c_smbus_read_byte_data(bme280_i2c_client,0xE4);

tempH4b = i2c_smbus_read_byte_data(bme280_i2c_client,0xE5);

tempH5a = i2c_smbus_read_byte_data(bme280_i2c_client,0xE6);

calib_H6 = i2c_smbus_read_byte_data(bme280_i2c_client,0xE7);

calib_H4 = (tempH4a<<4) | (tempH4b&0x0F);

calib_H5 = (tempH5a<<4) | ((tempH4b&0xF0)>>4);

if(calib_T2 > 32767)

{

calib_T2 -= 65536;

}

if(calib_T3 > 32767)

{

calib_T3 -= 65536;

}

printk("BME280 Calib Values\n");

printk("T1:%d\nT2:%d\nT3:%d\nP1:%d\nP2:%d\nP3:%d\nP4:%d\nP5:%d\nP6:%d\nP7:%d\nP8:%d\nP9:%d\n",calib_T1,calib_T2,calib_T3,calib_P1,calib_P2,calib_P3,calib_P4,calib_P5,calib_P6,calib_P7,calib_P8,calib_P9);

/* Sensor Initilatization Sequence */

i2c_smbus_write_byte_data(bme280_i2c_client, 0xf5, 5<<5);

i2c_smbus_write_byte_data(bme280_i2c_client, 0xf4, ((5<<5) | (5<<2) | (3<<0)));

return ret;

/* Error Handling on Failure */

KernelError:

device_destroy(bme280_class, bme280_device_number);

FileError:

class_destroy(bme280_class);

ClassError:

unregister_chrdev(bme280_device_number, DRIVER_NAME);

return (-1);

}

/* Driver Uninitialization routine */

static void __exit bme280Exit(void)

{

printk("bme280_deviceDriver - Goodbye, Kernel!\n");

i2c_unregister_device(bme280_i2c_client);

i2c_del_driver(&bme280_driver);

cdev_del(&bme280_device);

device_destroy(bme280_class, bme280_device_number);

class_destroy(bme280_class);

unregister_chrdev_region(bme280_device_number, 1);

}

/*Linking Init and Exit Functions to Driver Structure */

module_init(bme280Init);

module_exit(bme280Exit);

User space application build process and usage

/* General Includes */

#include<stdio.h>

#include<stdlib.h>

#include<fcntl.h>

#include<unistd.h>

#include<sys/ioctl.h>

#include<signal.h>

#include<unistd.h>

#include<time.h>

/* IOCTL Interface Definition */

#include "config.h"

/* Char Device Variable */

int file_desc;

/* Log File */

FILE *fptr;

/* SIGINT Signal Handler */

void exitHandler(int sig_num)

{

signal(SIGINT, exitHandler);

printf("\nExit Initiated");

close(file_desc);

fprintf(fptr,"\n");

fclose(fptr);

printf("\nShutdown Procedures Complete.Bye\n");

exit(0);

}

/* IOCTL Userspace Call for Pressure */

int ioctl_pressure(int file_desc, int32_t *msg)

{

int ret_val;

ret_val = ioctl(file_desc, IOCTL_PRESSURE,msg);

return ret_val;

}

/* IOCTL Userspace Call for Temperature */

int ioctl_temperature(int file_desc, int32_t *msg)

{

int ret_val;

ret_val = ioctl(file_desc, IOCTL_TEMPERATURE,msg);

return 0;

}

/* IOCTL Userspace Call for Humidity */

int ioctl_humidity(int file_desc, int32_t *msg)

{

int ret_val;

ret_val = ioctl(file_desc, IOCTL_HUMIDITY,msg);

return 0;

}

/* Main Routine */

int main(void)

{

int ret_val,i;

int32_t recv_msg;

time_t ltime;

struct tm result;

char time_string[32];

float temperature,pressure,humidity;

/* Register SIGINT Handler */

signal(SIGINT, exitHandler);

/* Open Char Device */

file_desc = open(DEVICE_FILE_NAME,0);

/* Log File Open */

fptr = fopen("data.log","a");

if(file_desc<0)

{

printf("Device Open Failed for %s\n",DEVICE_FILE_NAME);

exit(-1);

}

printf("Logging started...\n");

/* Infinite Execution Block */

while(1)

{

/* Current Timestame Retrieval */

ltime = time(NULL);

localtime_r(&ltime, &result);

asctime_r(&result, time_string);

i=0;

/* Format Timestamp for Log Print */

while(time_string[i]!='\0')

{

if(time_string[i]=='\n')

time_string[i]=0;

i++;

}

/* Print Timestamp to Log File */

fprintf(fptr, "%s : ",time_string);

/* Retrieve Temperature and Print to Log File */

ioctl_temperature(file_desc,&recv_msg);

temperature = (float)recv_msg/100.0;

fprintf(fptr, "Temperature:%f ",temperature);

/* Retrieve Pressure and Print to Log File */

ioctl_pressure(file_desc,&recv_msg);

pressure = (float)recv_msg/100.0;

fprintf(fptr, "Pressure:%f ",pressure);

/* Retrieve Humidity and Print to Log File */

ioctl_humidity(file_desc,&recv_msg);

humidity = (float)recv_msg/1024.0;

fprintf(fptr, "Humidity:%f\n", humidity);

/* Delay for 5 Seconds */

sleep(5);

}

}

IOCTL Header file:

#ifndef CHAR_CONFIG_H

#define CHAR_CONFIG_H

#include <linux/ioctl.h>

#define MAGIC_NUM 225

//IOCTL interface prototypes

#define IOCTL_TEMPERATURE _IOWR(MAGIC_NUM, 0, int32_t *)

#define IOCTL_HUMIDITY _IOWR(MAGIC_NUM, 1, int32_t *)

#define IOCTL_PRESSURE _IOWR(MAGIC_NUM, 2, int32_t *)

//Device file interface

#define DEVICE_FILE_NAME "/dev/bme280"

#endif

Makefile:

# All the Parameters can be over-ridden while Make call from the Terminal. Default values are assigned to match with the currnet project files.

# Note: The make recepies are designed in such a way assuming the make invoke is done from superuser(root)

# Name of the Userapp file.

USERFILE ?= userapp

# Name of the Kernel File

KERN_NAME ?= bme280

# Name of the Data Log file used for storing sensor values

DATAFILE ?= data.log

# Default Text Editor for displaying the log file

TEXT_EDITOR ?= nano

# Kernel Module Name Definition

obj-m := $(KERN_NAME).o

# Compiles the Kernel and Userapp files

all:

make -C /lib/modules/$(shell uname -r)/build M=$(shell pwd) modules

gcc -o $(USERFILE) $(USERFILE).c

# Compiles the Kernel code alone

kern:

make -C /lib/modules/$(shell uname -r)/build M=$(shell pwd) modules

# Compiles the Userapp alone

user:

gcc -o $(USERFILE) $(USERFILE).c

# Cleans the directory (Won't remove the log file)

clean:

make -C /lib/modules/$(shell uname -r)/build M=$(shell pwd) clean

rm userapp

# Installs the Compiled Kernel Module into Kernel Space

install:

sudo insmod $(KERN_NAME).ko

# Removes the loaded module from the Kernel Space

remove:

sudo rmmod $(KERN_NAME).ko

# Runs the Userapp Application

run:

sudo ./$(USERFILE)

# Deletes the Sensor Log generated by Userapp application

log_clean:

rm $(DATAFILE)

# Displays the Log generated by the Userapp application

log_display:

nano $(DATAFILE)

MAKE COMMANDS:

All the Operations are defined as Makefile recepies to make it easier for the execution

Following are the various recepies defined for compilation and execution

- all - Compiles Kernel and User Space Code

- kern - Compiles Kernel Space Code

- user - Compiles User Space Code

- install - Installs Kernel Module into Kernel Space

- remove - Removes Kernel Module from Kernel Space

- log_display - Displays the Log generated by User Space application

- log_delete - Deletes the Log generated by the User Space application

- log_clean – will delete the data log file

- clean – erases the all the executables and compiled files

- run - Runs the User Space Application